# Project Report

# CSE331L: Microprocessor Interfacing & Embedded Systems Lab

## Faculty: Kazi Safkat Taa Seen (KSE)

## Lab Instructor: Jannat Sultana

## Section: 04, Fall 2025

## Project Title: Smart Greenhouse Cultivation, Irrigation & Safety Monitoring System

## Smart Greenhouse Cultivation, Irrigation & Safety Monitoring System

## Team Members:

| Name | Student ID |
| --- | --- |
| Md. Nasim Ahmed | 2232936042 |
| Md. Asraful Hossain Riyan | 2232845642 |
| Hasinur Rahman Rohan | 1912835642 |
| Hassan Abdirahman Abdullahi | 2212938045 |

# Project Title

Smart Greenhouse Cultivation, Irrigation & Safety Monitoring System.

# Abstract

Greenhouse cultivation significantly improves crop yield by maintaining ideal environmental conditions. However, manually monitoring temperature, humidity, and soil moisture is labor-intensive and prone to human error. This project addresses these challenges by implementing a low-cost, automated Smart Greenhouse system using the STM32F103C8T6 (Blue Pill) microcontroller. The system performs real-time environmental sensing, automated irrigation control based on soil conditions, and safety monitoring.

The prototype utilizes a DHT11 sensor for temperature and humidity, an analog soil moisture sensor to trigger automatic irrigation via a servo motor, and a flame sensor to detect fire hazards immediately. Additionally, an ultrasonic sensor is employed to detect motion near the greenhouse entrance for security. A 0.96-inch OLED display provides a local user interface, while a 4x4 keypad allows for manual control and menu navigation. Testing confirmed that the system successfully automates irrigation when soil is dry and triggers instant audio-visual alarms during safety events, demonstrating a practical application of embedded systems in smart agriculture.

# Introduction and Motivation

Introduction: Greenhouses regulate climate for optimal crop growth, but manual management is inefficient. This project introduces a "Smart Greenhouse" prototype that uses electronic sensors to continuously monitor environmental conditions and safety hazards, bridging the gap between traditional farming and modern automation.

Motivation:

- Real-World Problem: Small-scale farmers often rely on guesswork for soil moisture and temperature, leading to crop stress. Additionally, fire hazards or intrusions may go unnoticed without automated alerts.

- Relevance: This system offers an affordable entry into "Smart Agriculture," reducing physical labor while providing immediate visual feedback and safety warnings.

- Course Relation: 
GPIO & Interrupts: Used for the Keypad scanning and Flame sensor (EXTI) for immediate response.
ADC (Analog-to-Digital Conversion): Used to interpret analog signals from the Soil Moisture sensor. 
Timers & PWM: Essential for controlling the Servo motor (valve) and reading the Ultrasonic sensor.
I2C Protocol: Implemented for efficient communication with the OLED display.

# System Overview

The system is built around the STM32 Blue Pill microcontroller, which acts as the central brain. It continuously polls data from various sensors to monitor the greenhouse environment.

The overall architecture and signal flow are summarized in Figure 1.

Figure 1: System block diagram.

Feature talk:

• Environmental monitoring - Displays real-time temperature/humidity (DHT11) and soil moisture level (ADC) on the OLED.

• Automatic irrigation decision - When soil moisture falls below the threshold, the servo opens the valve; it closes when moisture returns to normal.

• Fire/overheating alert - Flame sensor input is handled via interrupt to ensure fast response; buzzer and OLED warning activate immediately.

• Intrusion/motion detection - Ultrasonic sensing detects nearby movement; the system can switch to security mode and raise alerts.

• On-device user interface - Keypad-driven menu enables viewing pages, manual overrides, and configuration without a PC.

# Implementation Details

## Hardware Implementation

Table 1 lists the final hardware modules used in the prototype along with justification 	and an estimated cost breakdown (prices may vary by vendor).

Table 1: Final hardware module list and estimated cost breakdown.

| Hardware Module | Purpose | Interface | Qty | Est. Unit Cost (BDT) | Justification |
| --- | --- | --- | --- | --- | --- |
| STM32F103C8T6 (Blue Pill) | Main controller for sensor interfacing and control logic | GPIO/ADC/Timers/I2C | 1 | 350 | Low-cost ARM Cortex-M3 MCU with rich peripherals |
| ST-LINK V2 | Programming and debugging interface | SWD | 1 | 400 | Reliable flashing and debugging for STM32 |
| DHT11 | Temperature and humidity measurement | Single-wire GPIO | 1 | 80 | Simple and commonly available sensor |
| Soil Moisture Sensor (analog) | Soil condition monitoring for irrigation | ADC | 1 | 120 | Analog output allows threshold-based control |
| Flame/Heat Sensor | Fire/overheating detection | Digital GPIO (EXTI) | 1 | 70 | Fast event detection using interrupt |
| Ultrasonic Sensor (HC-SR04) | Motion/distance sensing | Timer/GPIO | 1 | 130 | Inexpensive distance sensor for security feature |
| 0.96" OLED Display (SSD1306) | User display for values, menus, and alerts | I2C | 1 | 350 | Low power, high contrast; simple I2C wiring |
| 4x4 Matrix Keypad | User input for menu and control | GPIO matrix scan | 1 | 120 | Simple UI input device without extra components |
| Servo Motor (SG90/MG90S) | Valve/door actuation | PWM | 1 | 250 | Direct position control suited for valve mechanism |
| Buzzer | Audible alarm output | GPIO | 1 | 20 | Immediate alert with minimal circuitry |
| Breadboard + Jumper Wires | Prototype interconnections | N/A | 1 set | 220 | Rapid prototyping and testing |
| Misc. components | Signal conditioning and safe driving | N/A | 1 set | 80 | Improves reliability (resistors, transistor, connectors) |
| Power supply / 5V source | Stable power for sensors and actuators | N/A | 1 | 200 | Required for stable sensor readings and servo operation |
| Estimated Total |  |  |  | 2390 |  |

The prototype was assembled on a breadboard with the Blue Pill as the central 		board. Sensors were placed on the periphery to simplify wiring, and the OLED was 	mounted on the top side for easy viewing. The servo was powered from a stable 5V 	source (with common ground to the STM32) to avoid brown-outs during movement. 
	Figure 2 illustrates a high-level wiring/interface mapping used in the prototype.

Figure 2: High-level wiring/interface diagram (prototype).

## Software Implementation

The firmware was implemented in C using the STM32 HAL libraries (STM32CubeMX/CubeIDE or Keil uVision with ARMCLANG) and programmed via ST-LINK. 
A modular driver approach was followed: separate functions/modules handle OLED (SSD1306 over I2C), DHT11 timing, keypad scanning, ultrasonic measurement, servo PWM control, and ADC sampling for soil moisture.

Control is organized as a simple state machine with periodic sensing and event-driven interruptions. In each loop iteration, the system scans the keypad to update the active page/mode, reads sensors, evaluates thresholds, and updates actuators. Safety events (flame detection) are handled with higher priority to ensure fast alarm activation. Figure 3 shows the high-level control flow.

Figure 3: High-level control flow.

Interfaces and protocols used in the prototype:

• I2C: OLED display communication (SSD1306).

• ADC: Soil moisture sensor analog sampling (with optional averaging/filtering).

• GPIO matrix scanning: Keypad row/column scanning with pull-ups and debouncing.

• Timers / PWM: Servo control (PWM) and ultrasonic echo timing (input capture or microsecond timing).

• EXTI interrupt: Flame sensor alert for immediate response.

# Discussion

- We successfully integrated multiple sensors into a cohesive dashboard. The I2C OLED provides clear data, and the safety interrupts (flame detection) react instantly. The servo motor integration proves the system's ability to physically actuate based on commands.

- Expected vs. actual outcomes:

All major features from the proposal (environment monitoring, flame alert, motion detection, and local UI) were implemented at prototype level.

- Key challenges and mitigation:

DHT11 timing sensitivity required careful microsecond delays and avoidance of blocking operations during sampling. Keypad scanning needed debouncing to prevent multiple detections per press. Servo actuation introduced supply noise, so a separate 5V supply (with common ground) and basic filtering were used to keep readings stable.

- Deviations from the original plan:

The proposal included water tank management and an automatic irrigation system. We were told not to implement them; thus, they were not bought or implemented. They had a prototype in the project as an option for water filling was represented as an alert/indicator which was modified accordingly.

- Limitations and future improvements:

The prototype does not include remote monitoring (Wi-Fi/GSM) or long-term data logging. Sensor accuracy is limited by DHT11 and low-cost analog probes, and the system requires calibration per soil type. 
Future work includes adding ESP8266/ESP32 connectivity, SD card logging, using higher-accuracy sensors (DHT22/SHT3x), implementing closed-loop control for irrigation, and designing a PCB for robust deployment.

# Appendix (optional)

Appendix A provides an example pin map used during prototyping. Pin assignments may be updated depending on the CubeMX configuration.

| Module | Signal(s) | Example STM32 Pin(s) |
| --- | --- | --- |
| Soil Moisture | Analog out | PA0 (ADC1_IN0) |
| DHT11 | DATA | PA1 |
| Flame Sensor | Digital out | PA2 (EXTI) |
| Ultrasonic | TRIG / ECHO | PA3 / PB1 |
| OLED (I2C) | SCL / SDA | PB6 / PB7 |
| Servo | PWM | PA8 (TIM1_CH1) |
| Buzzer | Digital | PB0 |
| Keypad | Rows / Columns | PB8–PB11 / PA4–PA7 |

Table 2: Example pin mapping used in the prototype.